OTA Design:

We will start the design by trying to make it single stage and single pole. We will using the folded cascode:

If the system is single pole, this means that the GBW reads 130⋅106, using the equation for GBW we find the gm1:

GBW=gm12πCL→gm1=GBW*2πCL=8.16 mS

This is a relatively high gm1. The main idea will be to keep the transistors small so that no more poles form on the system.

The gm equation reads:

gm=2IDSVGS-Vth

To get a high gm we can both increase the Ids or decrease the overdrive voltage. Increasing the Ids without touching the overdrive means to increase the transistor size. Decreasing overdrive without touching the current means increasing transistor size as well to compensate.

Here we see the first problem which is that the source of the input transistors are the drain of the active load M3.

We will start with a (comfortable) target source of 0.5V on the input sources. The drain of these transistors should be smaller than Vtp because the gate is 0VDC and the saturation rule reads:

VDG<-Vtp

So we will start with 0.3V.
We start by setting the length of the input transistors to Lin=200nm and iterate through their width. Assuming a Vov=200mV we should get IDS≅gm1*Vov2=800μA.

We found a close result for WL,in=130 μm:

We notice that both transistors need to be fed by M3 so we’ll choose M3 width such that it’s current is 1.86mA (L3=200nm):

So we choose W3=213.4 μm.

Next we need to set the output resistance.

The output resistance at each node is given by:

Rout=gm7ro7⋅ro5∥ro2∥gm9ro9r11

We need to achieve 1000 gain. Therefore, the Rout is given by:

Rout=124.6 kΩ

We will assume that ro2≈ro5 and therefore the equation reads:

Rout=gm7ro7⋅ro22∥gm9ro9r11

Moreover, we will assume that: gm7ro7⋅ro22≈gm9ro9r11 so that:

Rout=gm7ro7ro24

From DC point analysis we get:

ro2≈1kΩ

This will require that gm7ro7≅500 which is unreasonable for a single stage (usually 10 or 20).

Because this seems unreasonable to achieve we will aim for ro2=50kΩ wich means increasing the L by a factor of 50 and using the multiplier option on the transistor to get the desired width without increasing the parasitic impedance too much.

Start with L = 5mu and sweep on W = 50u and multiplier value Fin.

F_in = 108