EX2 – Wideband Linear Driver

We want to design the following driver:

We will concentrate on each stage, design for it and then integrate together.

Stages Overivew:

The leftmost stage (M7 R1) is a biasing stage responsible for setting the voltage of all the PMOS current sources of the next stages.

The input stage (M5 M6) is a simple buffering stage, because of the common drain M5. The DC level of vin is 0V and therefore we must increase it in order to be far enough from the rail voltages (0 and 1V). As such, we expect a gain close to 1.

The third stage (M4 M3) is a classical common source (M3) gains stage with a PMOS current source (M4). This gain will be responsible for the gain A3=gm3(r03|ro4.

The last stage is a class AB is the output stage. In order to buffer this stage from the previous stage we will ensure that no current flows through RF at DC. This can be done for example if we enforce that at both sides of RF V=VDD2=0.5V.

Output Stage Design:

To design the output stage we use the following subcircuit:

Notice that in this subcircuit the input gate was taken to be 0.5V. The PMOS and the resistor are disabled and a current source of 20μA was randomly chosen (if 20 μA is found to be OK for this stage then the stage will consume 10% of the overall allowed consumption of 200 μW).

We will use WN1=2μm and iterate through various values of L to find the L that gets us to Vout=0.5V @DC.

From the following figure, we see that the closest “valid” value that we can use is LN1=2.3μm. The precise voltage now is of less importance as we are going to fine tune it with the PMOS, but we write it for reference as V=467mV.

Next, the PMOS is simulated with the following subcircuit:

Notice that in this case the current has been inverted. Because PMOS are usually half as strong as NMOS we will start with WP1=4μm and iterate over LP1. We try to find the LP1 such that the drain voltage is VDD2.

The closest voltage we found was 656mV for 400nm.

Now we connect the PMOS and the NMOS together without the current source and without the resistor to find the drain voltage. The drain voltage was found to be 582mV. This means that the PMOS pulls stronger than the NMOS.

We increase the NMOS width by steps of 100nm in order to find the width for which the drain voltage is 0.5V.

We found that for WN1=2.1μm the drain voltage settles at 0.515V.

Now we set the resistor. The resistor needs to accommodate for the bandwidth needed. Looking from the output, we see RBro1r02, if RB is much smaller than r01||r02 then we will effectively see only RB.

In the case that RL is much lower than r01||r02, the bandwidth reads:

f3dB=12πCLRB

Solving for f3dB=1GHz and CL=10fF we get RB=15.9kΩ. Both transistors have ron~100kΩ so the assumption that RB is much smaller than r01||r02≅50 kΩ is not much justified. We will therefore try to use RB=23kΩ as in such case the resulting resistance is approximately 15.7kΩ.

To simulate this we use the following circuit:

The response is given next:

We see that approximately the BW is 1GHz and that the gain of the output stage is 8.5dB.

Common Source Stage (M3 M4).

The requirements for this common source stage are to have around 16.5dB gain and at least 1 GHz BW.

The main difficulty to solve here is that the output seen at this stage is resistive (RB) generating a loading effect that will hurt the gain of the amplifier.

We will try to analyze this circuit by assuming that the resistor RB simply ads in parallel to the overall circuit.

The plan will be to get to 0.5V at the drain of the transistors and replicate the current (increasing W) so that the gain increases.

To find 0.5V at the drain of the transistors we need not to include the resistor, because in the same way that was assumed for the output stage, the resistor will only affect the ac point.

We need to assume something about the input DC voltage to transistor M3. Because this transistor will be responsible for most gain, we should use a low voltage. However, as this voltage is the drain voltage from the input stage (M5 M6), using a low gate votlage will result in taking M5 out of conduction!

We will start with the trivial but convenient assumption that the gate of M3 is biased with 0.5V. Using this assumption, we wish to use the transistor at VGD=0 (which is clearly saturated).

The circuit used to simulate this stage is shown next:

Notice the disconnected PMOS and votlage source, and the added current source.  The starting current source is taken to 20μA and it will be changed later on. The first step is to find some L such that for a predefine W the drain voltage is 0.5V (this is the same as done before). We choose randomly WN2=2.1 μm and iterate over L’s.

We proceed with LN2 =2.5 μm.

Now we repeat for the PMOS. For the PMOS we invert the direction of the current source, disable the NMOS sources and enable only PMOS sources.

The gate of the PMOS is trivially biased to 0.5V. After a multidimensional (W and L) simulation we found a nice solution over WP2=3.8 μm and LP2=400nm:

This is the closest solution we found.

The next step is to connect the transistors and try to manually tune them to reach the desired 0.5V.

The closest solution we found was for LN2=1.8μm and WP2=3.9μm LP2=300n.

Now we will connect both stages together and find the overall gain. If more gain is needed we will increase the current of the common source stage.

The previous figure shows the DC point of the last two stages. We can se that the reisistor current is negligible with respect to the stages currents, indicating minimal effect on the DC point (as desired).

The AC response is shown next:

Net5 and net6 are the left and right nodes of the resistor Rb respectively. We can see that the common source stage suffers from low gain, probably from the loading of Rb that kills the output resistance of the stage.

We do see that the gains of the stages are multiplicative, meaning that we can take the common source stage and use it (there will be some gain reduction because of the output resistance loading from transistors M2 and M1).

In order to increase gain without paying high current, we can increase L and reduce Vov of the common source transistor M3.

We will use the following circuit:

The voltage source on the right node of R3 is set to 0.5V to resemble the output node DC point (approximately).

We decrease VGSN to 0.4V and find the W needed so that the left node of R3 sits at 0.5V DC. A close solution is found for WN2=4.3μm

This is interesting because it lets us use a smaller VGS while keeping the same DC point, leading to higher gain for the same current expenditure.

Now, we will iterate for various finger count values (which are simulated simply by multiplying both WN2 and WP2 by the same factor N.

The results of such plots are seen next:

It can be seen from the figure that increasing L increases the gain at the cost of GBW (because the parasitic capacitacnes increases). Smaller parasitic capacitances are achieved by making the absolute W and L values of the transistor smaller (while keeping the relationships). This trades with gain as it will decrease the output resistance (decreasing L). However as the BW increases quadratically, and the gain decreases linearly on transistor sizes, we should get a linear increase in GBW per line.

The last figure shows a much broader response as expected. The finishing of the response will be done later.

We will chose the highest gain (N=7). After some manual tunning we got:

With the parameters:

N = 7

LN2=1.8μm*N

WN2=2.4μm*N

LP2=200nm

WP2=1.8 μm*N

Then we connect the two stages together and get the following DC OP;

We can clearly see that the current flowing through the resistor isn’t negligible wrt to the output stage current. To fix this we will manually tweak the output PMOS to meet the 500mV mark.

And the corresponding ac line reads:

Second Venure:

We start by noticing that the output stage and the common source stage are two amplifiers. If we can make the miller capacitance seen at the input of the output stage, be much bigger than the output capacitance, then such capacitor will form the principal pole and will be responsible for the f3dB. Denoting such capacitance as CM=CgdA2 and the resistance seen at the input of the second stage as Rin2 we get the expression:

f3dB=12πRin2CgdA2

The gain of the common source stage is given by:

A1=gm1⋅Rin2

The gain of the second stage is given by:

A2=gm2⋅Rout

The GBW product is given by:

f3dB⋅A1⋅A2=gm12πCgd

We remember that this assumes that CgdA2≫CL.

The strategy will be as follows:

- Start with initial guesses for the transistor values.

- Set the output stage so that the output DC is on 0.5V (max swing).

- Set the common source stage so that its output is also at 0.5V (this ensures minimal resistor influence).

- Measure the gate to drain capacitance.

- Change the current on the common source stage to match the gm needed for GBW.

We start by minimal lengths and widths of 200 nm and with the following circuit:

We set the input and output voltage of the output stage to 0.5V and will sweep the W values to find a (trivially set) value of I=20 μA.
Because we aim for a high Cgd we choose the initial value L=1μm.

For the NMOS:

And for the PMOS

From the figures we set: WP1=9.2 μm and WN1=1.3 μm. This gives a current of 23.55 muA at the output stage.

The total

Third Venure:

We think of the output stage and the common source stage as two amplifiers connected one after the other. Without the resistance RB the gain of the second stage would be g2r02 were the tilde symbolizes the overall effect of the NMOS and PMOS connected together.

The feedbacked output stage can be replaced with a input resistor to the ground with resistance RBg2r02 and an output resistance RB to ground.

If we assume that the total output resistance from the commons source stage is similar to the output resistance of the output stage, and we assume that the input capacitance to the output stage is several dozens of femto farads (which can be designed for), then, the dominant pole will be at:

f3dB=12πCin2ro1

The total gain will be at:

Av=gm1ro1gm2ro2

The GBW of the system is given by:

GBW= gm1ro1gm2ro22πCin2ro1=gm1gm2ro22πCin

Notice that this development doesn’t assume anything about the relationship between RB and roi for i=1/2.

Note: 
If the achieved input capacitance to the output stage is of the same order or smaller than the output capacitance, then the dominant pole will sit at:

f3dB=12πCLro2

And the GBW will read:

GBW= gm1ro1gm2ro22πCLro2=gm1gm2ro12πCL

This result is less desired as ro1 depends on RB and gm2ro2 and will give a complex dependency to design for.

The nice thing about our design, is that the RB resistance can be included at the output simply by noticing that:

ro2=roP||roN||RB

Another simplification that we can make is to notice that once we use the miller resistance, we have the concatenation of two amplifiers.

We will start by using a high RB. In this case, RF will affect little our gain and GBW.

We will choose it RF=500kΩ, we will also use Linit=2μm for all transistors.

We’ll start from the output stage and aim for a 0.5 VDC for maximal swing. 
The main consideration for gain split between the output stage and common source stage should be the reduction of resistance RB at the miller equivalent RBgs2. This means that we should achieve much of the gain on the common source so the miller resistance isn’t to low.

We can aim for 25 dB on the common source and 0 dB on the output stage. This is a good idea because if achieved, it will result in the miller resistance on the input side to be RG!

We will begin by the output stage with the following circuit:

V10 secures a gate voltage of 0.5V, which was set from symmetry considerations (both PMOS and NMOS provide gain on the output stage so there is no reason to favor one over the other). V11 set’s the output voltage on 0.5V and we will aim to set a WN1 and a WP1 that get’s the PMOS and the NMOS to the same current.

The NMOS currents are presented next:

The PMOS currents are presented next:

So we choose WN1=1.1μm and WP1=9.0μm.

Disabling votlage source V11 we confirm the dc biasing point of the output stage. The output DC point is 493mVDC on this case, which means a current of ~12nA on the resistor, which is negligible w.r.t the 12μA current present on the transistors.

Fourth Venture:

We will assume that the rout of the output stage (M1-M2) combined with the output capacitors form the pole that we are seeking to get.

We get:

rout=1f3dB2πCL=15.9 kΩ

The RF capacitance reflects both output and input of the output stage, to the output stage it reflects approximately as RF and to the input it reflects approximately as RF(gm1+gm2)(r01|r02.

Using this notation we get:

rout=ro1r02RF

If RF is big enough, we will get rout=ro1||ro2.

The strategy will be as follows:

- Find W and L values so that both transistors are at Vout=0.5V with the same current.

- Increase this current (by increasing the transistor widths, therefore more current) up to the point the output resistance is OK.

We will begin by the output stage with the following circuit:

We will start with minimal transistor lengths to support high current with low input capacitance (to be explained later on).

V10 secures a gate voltage of 0.5V, which was set from symmetry considerations (both PMOS and NMOS provide gain on the output stage so there is no reason to favor one over the other). V11 set’s the output voltage on 0.5V and we will aim to set a WN1 and a WP1 that get’s the PMOS and the NMOS to the same current.

The NMOS currents are presented next:

The PMOS currents are presented next:

On the same graph:

So we choose WN1=700nm and WP1=3μm.

After removing the V11 voltage source we found that the voltage achieved was 485mVDV on the output. 
We will increase the width of both transistors by a multiplicative factor N1 and find the overall output resistance (without considering RG).

The results are shown next:

Choosing N≅2.836 will give WN1=2μm  and WP1=8.5μm.

Next we check the AC response of the circuit, and verify that the design works.